Reactor Core Degradation Neural Network

Model Development & Optimization Reference Sheet

Candidate Features

All features drawn from NRC RVID v2.0.1 (974 samples after cleaning).

| Feature | Description | Physical Basis | Final Model |
| --- | --- | --- | --- |
| %Cu | Copper content (wt%) | Primary embrittlement driver | ✓ Included |
| %Ni | Nickel content (wt%) | Amplifies Cu effect at high fluence | ✓ Included |
| %P | Phosphorus content (wt%) | Minor contributor at high concentrations | ✗ Excluded |
| %S | Sulfur content (wt%) | Minor embrittlement contributor | ✓ Included |
| log(fluence) | Log-transformed neutron fluence at EOL 1/4T | Log transform per NRC RG 1.99 — embrittlement response is logarithmic | ✓ Included |
| RTndt(u) | Initial unirradiated reference temperature | Material-specific baseline toughness | ✓ Included |
| Cu×log(f) | Interaction term: Cu content × log(fluence) | Explicitly encodes Cu-fluence synergy | ✗ Excluded |

Note: %P and Cu×log(f) were excluded by the final Optuna search. %P has low signal at typical RVID concentrations. Cu×log(f) was learned implicitly by the 3-layer architecture without needing to be provided explicitly.

Evaluation Metrics

| Metric | Measures | Why It Matters Here | Regulatory Target |
| --- | --- | --- | --- |
| RMSE | Error weighted toward large misses | Catches dangerous mispredictions near PTS limits | Minimize |
| MAE | Straight average error | Typical day-to-day accuracy in °F | Minimize |
| R² | Variance explained vs. blind guess | Model-to-model comparison | Maximize |
| Std Dev of Residuals | Consistency of prediction errors | Directly tied to NRC margin requirements | ≤28°F welds / ≤17°F base metal |

Baseline Models

| Model | RMSE | MAE | R² | Notes |
| --- | --- | --- | --- | --- |
| Linear Regression (raw fluence) | 32.11°F | 23.14°F | 0.676 | 80/20 split |
| Linear Regression (log fluence) | 25.37°F | 18.57°F | 0.798 | Single transform, ~7°F gain |

The log-fluence transform alone — replacing raw fluence with its natural log — improved R² by 12 points and reduced RMSE by 6.74°F. This validated the physical assumption that embrittlement scales logarithmically with dose and carried forward into all subsequent models.

Initial PyTorch MLP

Architecture

- 2 hidden layers: 100 → 50 neurons

- ReLU activations, Adam optimizer, lr=0.001, 1,500 epochs

- 5-fold cross-validation, StandardScaler normalization

- Features: %Cu, %Ni, %P, %S, log(fluence), RTndt(u)

| RMSE | MAE | R² | Std Dev |
| --- | --- | --- | --- |
| 11.40°F | 6.84°F | 0.9589 | 11.37°F |

The jump from R²=0.798 (log-linear) to R²=0.959 reflects the MLP capturing non-linear Cu×fluence interactions that linear models cannot represent. Std Dev of 11.37°F already meets both NRC thresholds.

Three-Branch Optimization

The codebase was split into modular files (data.py, model.py, train.py, main.py) and three independent branches explored optimization axes in parallel.

Branch 1: Hyperparameter Tuning (train.py)

Optuna searched across: learning rate, optimizer (Adam/SGD/RMSprop), momentum, weight decay, dropout rate, scheduler (cosine/step/exponential/none), weight initialization, and epochs. GPU acceleration (RTX 4080) and gradient clipping (max_norm=1.0) were added. Hundreds of trials completed across multiple runs.

- Best RMSE achieved: 10.71°F

- Best config: SGD, lr=0.000231, momentum=0.886, cosine scheduler, kaiming init, 2,339 epochs

- Takeaway: modest gain — confirmed initial MLP was already near its performance ceiling

Branch 2: Feature Engineering (data.py)

Explored removing low-signal features (%P, %S) and adding the Cu×log(fluence) interaction term to explicitly encode the known physical synergy between copper content and radiation dose.

| Metric | Original | Feature Engineered | Delta |
| --- | --- | --- | --- |
| RMSE | 11.48°F | 11.33°F | -0.15°F |
| MAE | 6.92°F | 6.52°F | -0.40°F |
| R² | 0.9584 | 0.9605 | +0.002 |
| Std Dev | 11.46°F | 11.33°F | -0.13°F |

Key insight: removing %P and %S reduced noise, and the Cu×log(f) interaction term significantly improved MAE, suggesting the explicit encoding helped the model on typical (non-outlier) vessels. These results directly informed the final Optuna run — all 7 features were included as candidates, letting Optuna determine which to keep. The final model retained %S but dropped %P and Cu×log(f), validating that a deeper architecture can learn the Cu×fluence interaction implicitly.

Branch 3: Architecture (model.py)

Explored variable layer depth (2–4 layers), layer sizes (32–256, funnel constrained), activation functions (ReLU/LeakyReLU/ELU), and batch normalization. Gains were comparable to the hyperparameter branch — individually modest, but the best-performing configurations (deeper networks, larger first layers) informed the search space for the final run.

Final Unified Optuna Run

All three axes — features, architecture, hyperparameters — were searched simultaneously. 1,278 trials over 8 hours on RTX 4080 GPU. %Cu always included, minimum 2 features required, funnel architecture constraint enforced.

Best Configuration

| Parameter | Category | Value |
| --- | --- | --- |
| Selected Features | Features | %Cu, %Ni, %S, log(f), RTndt(u) |
| n_layers | Architecture | 3 |
| first_layer_size | Architecture | 256 |
| activation | Architecture | ReLU |
| batch_norm | Architecture | False |
| optimizer | Hyperparameter | Adam |
| lr | Hyperparameter | 0.001627 |
| epochs | Hyperparameter | 2,546 |
| scheduler | Hyperparameter | Cosine |
| dropout_rate | Hyperparameter | 0.034 |
| weight_decay | Hyperparameter | 0.00579 |
| init | Hyperparameter | Kaiming |

Final Results Summary

| Model | RMSE | MAE | R² | Std Dev |
| --- | --- | --- | --- | --- |
| Linear Regression | 32.11°F | 23.14°F | 0.676 | — |
| Log-Fluence Linear | 25.37°F | 18.57°F | 0.798 | — |
| Initial MLP | 11.40°F | 6.84°F | 0.9589 | 11.37°F |
| Final Optimized MLP | 10.25°F | 5.85°F | 0.9678 | 10.24°F ✓ |

Std Dev of 10.24°F meets both NRC thresholds (≤28°F welds, ≤17°F base metal). The modest gain from initial MLP to final optimized model (1.15°F RMSE) reflects that the initial architecture was already near the performance ceiling for this dataset size. The 1,278-trial search confirmed this empirically.